Pseudocode

Algorithm 1: Rolling-horizon simulation and post-processing

INPUT:

Hourly wholesale prices, grid tariffs, PV generation, and building loads

EV and stationary-battery parameters

Mobility schedules, trip distances, and departure-energy requirements

Carbon-emission factors, network parameters, and resilience-event data

EV participation modes M = {0, 1, 2}

Use cases U = {UC1, UC2}

FOR each use case u in U:

Construct hourly EV location, connection, travel, and trip-energy schedules

Set initial EV and stationary-battery states

FOR each EV mode m in M:

Initialise the EV and stationary-battery states for this run

FOR each hour t = 1, ..., 8,760:

Build the 24-hour optimisation horizon starting at hour t

Load the relevant price, generation, demand, and mobility inputs

Solve the optimisation problem subject to:

EV mobility and departure-energy requirements

EV battery and charger limits

Stationary-battery operating limits

Node and community energy-balance constraints

P2P trading and price-feasibility conditions

V2G availability and activation constraints

Participation-mode restrictions

Implement decisions for hour t only

Update EV and stationary-battery states

Record hourly dispatch, prices, trades, payments, and state variables

END FOR

Verify hourly energy balances and operating constraints

Calculate the EV owner's payoff and individual-rationality status

Store the results for use case u and mode m

END FOR

Compare participating-mode payoffs with the defined outside option

Identify which modes satisfy the individual-rationality condition

Retain mode-specific results for reporting

Calculate stakeholder payoffs, system costs, social welfare,

emissions, and operational KPIs

FOR each admissible coalition S:

Re-solve the optimisation for coalition S

Calculate and store the characteristic value v(S)

END FOR

Calculate Shapley, τ-value, and AL allocations

Test core feasibility and the specified clan-game conditions

Run the defined resilience events and sensitivity cases

END FOR

Aggregate hourly results into monthly and annual outputs

Export hourly, summary, coalition, allocation, and stress-test tables