# Pricing Electric-Vehicle Flexibility in Local Electricity Markets: A Mobility-Aware P2P–V2G Market Design

# Supplementary Material S1

# Model Equations and Term Definitions

Numbering convention. The manuscript equation numbers Eq. (1)–Eq. (62) are retained. Sub-equations are labelled as in the methodology where needed (14a–b, 15a–b, 16a–b, 31a–c, 49a–b, and 50a–b).

Notation and scope. The index t denotes an hourly interval, n a node/prosumer, e an EV owner, and s a trip session or stress event as indicated by context. Unless stated otherwise, power is in kW, energy in kWh, prices in EUR/kWh, and monetary payoffs in EUR.

# Market inputs and calibrated profiles

## Eq. (1): Wholesale-price unit conversion

λᵂt=PriceDA,t/1000

Purpose: Converts the day-ahead price from EUR/MWh to EUR/kWh.

Term interpretation: λᵂtis the wholesale price at hour t; PriceDA,tis the day-ahead price in EUR/MWh; division by 1,000 converts it to EUR/kWh.

Literature basis:  (ENTSO-E Transparency Platform [Data set], 2025).

## Eq. (2): PV-Generation

EPVn,t=It⋅Α*0,18

Purpose: Scales a normalized hourly PV-Generation profile to each prosumer's installed PV capacity (PV system area).

Term interpretation: EPVn,t is PV energy generated at node n in hour t; It Solar irradiance (Wm2), representing the solar resource available for photovoltaic electricity generation. The dataset concerning Solar Radiation availability by region, month, and day is obtained from (Koundouris, et al., 2017). The city of Athens is selected as the reference region; Α is the PV system area in (m2). The efficiency of the PV is estimated at 18%.

Literature basis (BS EN 15316-4-3:2017, 2026)

## Eq. (3): Load-profile scaling

Ebase,t=Ln,t=Eᴬⁿⁿn* LS̄n,t

Purpose: Creates a residential or commercial hourly load from a normalized profile and annual demand.

Term interpretation: L_{n,t} is electricity demand per node n and hour t; Eᴬⁿⁿ_n is annual demand per node n; LS̄n,t is the normalized hourly load share

Literature basis: Baseline electricity consumption (kWh), representing the conventional household demand profile derived from historical consumption data and reflecting typical residential electricity usage patterns. Based on the estimated quantities of electricity consumption in residential households for the 25-year period 2025–2050, according to the National Energy and Climate Plan (NECP), (Greek Ministry of Energy, 2024) and (Koundouri, et al., 2025),  the total annual electricity consumption is available. This annual total is allocated to a typical daily load profile (load shape) on an hourly basis throughout the entire year, representing a typical household from (Karamanos, et al., 2019).

# EV mobility, energy states, and participation

## Eq. (4): Physical EV availability

The indicator of the physical EV availability is a binary one for the connectivity or not of EV to the node n , at time t:

αe,n,t= 1, with EV connected to node n, at time t0, if EV is not connected to node n, at time t (4)

Purpose: Separates physical connection from willingness to participate.

Term interpretation: a_{e,n,t} is a binary connection indicator: it equals 1 when EV e is connected at node n during hour t and 0 otherwise. It describes physical availability, not willingness to participate.

Literature basis: (Kempton & Tomic, 2005);  (Sortomme & El-Sharkawi, 2011).

## Eq. (5): Scheduled hourly distance

de,t= de,s*1 {t∈trip session, s }

Purpose: Assigns the planned travel distance to the correct hourly travel block.

Term interpretation: de,t is distance travelled by EV e during hour t; de,s is the distance assigned to trip session s; the indicator equals 1 only during that trip session (s).

Literature basis:  (Small & Verhoef, 2007);  (Sortomme & El-Sharkawi, 2011).

## Eq. (6): Scheduled trip-energy requirement

Ee,ttrip=κev* de,t

Purpose: Converts scheduled kilometres into traction-energy withdrawal.

Term interpretation: Ee,ttrip is traction energy used by EV e in hour t (KWh); κev is its energy consumption per kilometer (KWh/Km); de,t is the distance travelled in that hour (Km).

Literature basis:  (Small & Verhoef, 2007);  (Sortomme & El-Sharkawi, 2011).

## Eq. (7): Departure indicator

δe,tdep= 1, if t is a departure hour of EV0, otherwise

Purpose: Activates departure-SOC and mobility constraints only at departure times, since the departure indicator δe,tdep determines when it must be verified that sufficient energy (SOC level) is available for the EV's mobility requirements.

Term interpretation: δe,tdep is a binary indicator equal to 1 at an EV departure hour and 0 at other times.

Literature basis:  (Sortomme & El-Sharkawi, 2011).

## Eq. (8): Required energy at departure

Ee,tEV≥ δe,tdep*( Ee,strip+ Eebuf )

Purpose: Guarantees the planned trip plus a range-security buffer.

Term interpretation: Ee,tEV is stored EV energy at departure; Ee,strip is the energy required for the relevant trip; Eebuf is the additional range-security buffer. The constraint is activated by δe,tdep.

Literature basis: (Kempton & Tomic, 2005);  (Sortomme & El-Sharkawi, 2011).

## Eq. (9): EV battery-state transition

Ee,  t+1EV= Ee,tEV+nech* Pe,tch*Δt- Pe,tdisch*Δtnedisch-Ee,ttrip

Purpose: Updates EV stored energy after charging, discharging and travel.

Term interpretation: Ee,  t+1EV and Ee,tEV are consecutive battery-energy states; nech and nech are charging and discharging efficiencies; Pe,tch and Pe,tdisch are charging and discharging power; Δt is the time-step duration; Ee,ttrip is traction energy withdrawn for travel.

Literature basis: (Kempton & Tomic, 2005);  (Sortomme & El-Sharkawi, 2011).

## Eq. (10): EV battery and power bounds

Eemin≤ Ee,tEV,   stored ≤ Eemax (a)

Purpose: Enforces battery limits, charger limits and physical connection.

Term interpretation: Eemin and Eemax are respectively the minimum and maximum permissible EV battery energy level (kWh), where Ee,tEV,   stored is the restored energy level during time t (battery capacity constraint).

0≤ Pe,tch ≤ ae,n,t*Pech,max (b)

It is about EV charging power constraint and

0≤ Pe,tdisch ≤ ae,n,t*Pedisch,max (c)

Is about EV discharging power constraint

he EV battery energy and power constraints ensure that the stored energy remains within permissible operating limits, while charging and discharging activities are restricted by the vehicle's availability at an eligible network node and its maximum power ratings. These constraints preserve battery operational feasibility and regulate EV participation in local energy trading and vehicle-to-grid flexibility services.

Literature basis:  (Sortomme & El-Sharkawi, 2011).

# Prosumer storage and local energy allocation

## Eq. (11): Stationary-battery Prosumer’s BESS transition

En,t+1BESS= En,tBESS+nnBESS, ch* Pn,tBESS,ch*Δt- Pn,tBESS,disch*ΔtnnBESS, disch

Purpose: Tracks the residential/commercial prosumer battery.

Term interpretation: En,tBESS  and En,t+1BESS is stationary-battery (BESS) energy stored, in (KWh), at time t and t+1; nnBESS, ch and nnBESS, disch are its charging and discharging efficiencies; Pn,tBESS,ch and Pn,tBESS,disch are its charging and discharging power of the battery; Δt is the time-step duration of the time interval in (h). n, is the node of the local energy network.

The stationary battery energy balance equation describes the intertemporal evolution of stored energy as a function of charging and discharging power, accounting for the corresponding conversion efficiencies. Unlike mobile EV batteries, stationary storage systems are not subject to trip-related energy consumption, allowing their available capacity to be allocated to local energy trading, renewable energy integration, and flexibility provision, subject to operational constraints.

## Eq. (12): Stationary-battery bounds

To keep Battery Energy Storage System (BESS) of prosumer within capacity and converter ratings, optimization constraints, the model determines the battery’s state of charge and how quickly it charges or discharges, while ensuring that it remains within these technical limits.

EnB, min≤ En,tBESS≤ EnB,max (12a)

where En,tB

Is the restored energy level of the prosumer’s (n) battery at time t in KWh

0≤ Pn,tBESS,ch≤ PnB,ch,max (12b)

where Pn,tBESS,ch is the charging power of prosumer n at time t in KW

0≤ Pn,tBESS,disch≤ PnB,discch,max (12c)

## Eq. (13): Direct PV self-consumption

To measure PV generated energy which is self-consumed locally in the node of prosumer n, before export

En,tPV,self=min⁡{Gn,t, Ln,t+ Pn,tB,ch+Pn,tEV,ch}Δt (13)

Where:

Gn,t power generated by PV at time t

Ln,t load of building (prosumer) consumption

Pn,tB,ch charging power of BESS prosumer's battery

Pn,tEV,ch charging power of EV battery, Δt: time step (1 hour)

## Eq. (14a) / Eq. (14b): Residual surplus and deficit

To identify the prosumer's tradable surplus and unmet local demand at node of prosumer n, considering the PV generation, the consumption load of prosumer’s building and the battery operation (building BESS of prosumer):

Sn,t=[ Gn,t+ Pn,tB,disch-Ln,t- Pn,tB,ch]+ (14a)

Dn,t=[ Ln,t+ Pn,tB,ch-Gn,t- Pn,tB,disch]+ (14b)

Where + is the positive fact function, meaning that if the result is negative, it is recorded as zero, since that for the same time t, it is not possible to have positive surplus and deficit simultaneously.

# Rolling-horizon statistics and charging target

## Eq. (15a) / Eq. (15b: Rolling wholesale-price statistics

Within the rolling horizon, Eq (15) calculates the rolling wholesale-price statistics.

μtw=1H*tt+H-1λtw (15a)

Where: λtw, the wholesale price in €/ KWh at time t, H the time window of the rolling 24 hrs (from t to t+23),  μtw, the rolling mean of the wholesale price for this time window H.

The standard deviation of wholesale price in the rolling (t, t+23) time window gives the distance of the hourly price to the mean  μtw:

σtw = 1H*tt+H-1(λtw -  μtw )2  (15b)

The standard deviation is used to define the low and high thresholds. and, as a consequence to define low charging and high discharge options.

## Eq. (16a) / Eq. (16b): Price thresholds

As a consequence of the previous Eq (15b), Eq (16a) and Eq(16b) define low charging and high discharge options:

λtlow=μtw- σtw (16a)	 λthigh=μtw+ σtw (16b)

## Eq. (17): Desired EV charging energy

The desired EV charging energy in Eq (17) represents how much energy is required to EV battery at time t, considering both the charger and the energy required to get the targeted level of EV battery.

Qe,tdesired=minPech,maxΔt ,e,ttargE-e,tEvE+nech (17)

Where, Qe,tdesired the desired energy of charge in KWh; Pech,maxΔt, the maximum energy to pass through the charger in time step Δt ; e,ttargE the targeted energy for the EV battery ; e,tEvE the actual energy of the EV battery ;

e,ttargE-e,tEvE+ =max(0,  e,ttargE-e,tEvE),

the required step from actual to the desired EV battery energy level and

nech  :the charge coefficient lower than 1(means that the restored energy is less than the energy puchased by the prosumer/DSO

Literature basis:  (Palma-Behnke, et al., 2013);  (Sortomme & El-Sharkawi, 2011).

## Eq. (18): P2P energy purchased by the EV

qᴾ²ᴾ,ᵇᵘʸe,n,t=min{Qᵈᵉˢe,t,Sn,tΔt}⋅1{me≥1}

the P2P energy purchased by the EV, using local PV/BESS surplus before grid charging, when the EV participates is given by:

qe,n,tp2p, buy=minQe,tdesired , Sn,tΔt*1 (me≥1) (18)

Where Qe,tdesired is the energy desired for charging the EV in KWh

Sn,tΔt is the nodal energy surplus in KWh

Thanks to “min”, the minimum value will be chosen, so the purchased quantity will not exceed the desired charging EV level or the available nodal surplus. Since me≥1 the indicator 1 corresponds to the availability of P2P purchase (me# 0).

Literature basis:  (Long, Wu, Zhou, & Jenkins, 2017);  (Paudel, Chaudhari, Long, & Gooi, 2019).

## Eq. (19): Residual EV grid charging

qᵍʳⁱᵈ,ᵇᵘʸe,t=Qᵈᵉˢe,t-qᴾ²ᴾ,ᵇᵘʸe,n,t₊

Purpose: Supplies any remaining EV charging need from the external grid.

Term interpretation: qᵍʳⁱᵈ,ᵇᵘʸe,t is the energy purchased from the grid , the EV owner buys this quantity of energy (KWh); qᴾ²ᴾ,ᵇᵘʸe,n,t is the energy purchased from the local P2P market , the EV owner buys this quantity of energy (KWh); [x]₊ sets the result to zero if local purchases meet or exceed the desired charging amount (Positive-part operator).

The residual grid procurement equation determines the electricity quantity purchased from the main grid after accounting for the energy acquired through local peer-to-peer trading. The positive-part operator ensures non-negative grid purchases, allowing the main grid to cover any remaining electricity requirement not satisfied by the P2P market.

Literature basis:  (Sortomme & El-Sharkawi, 2011).

## Eq. (20): EV energy offered to P2P

qe,n,tP2P, sell≤minPedisch,max*Δt, nedisch *[ Ee,tEV- Ee,treserve ]+⋅1{me=2}*ae,n,t

Purpose: Limits peer export by power, reserve energy and participation mode.

Term interpretation: qe,n,tP2P, sell is the electricity (KWh)sold by EV owner in the local P2P market at node n during time period t;  Pedisch,max*Δt Maximum EV battery discharging energy (Pedisch,max is the power in KW); Ee,tEV- Ee,treserveis energy above the mobility reserve; nedischˢ EV battery discharging efficiency; me=2enables selling (V2G), 1me=2, the Binary indicator equal to 1 when the EV operates in mode 2 and 0 otherwise; ae,n,t , defines the availability of EV at node n.

The mobility-aware P2P selling constraint limits the electricity that an EV can supply to the local market according to its maximum discharging capability and the battery energy available beyond the required mobility reserve. Discharging efficiency accounts for conversion losses, while the operating-mode indicator restricts P2P sales to vehicles authorized to participate as energy suppliers. This formulation ensures that market participation does not compromise scheduled mobility requirements.

Literature basis: (Kempton & Tomic, 2005);  (Sortomme & El-Sharkawi, 2011).

## Eq. (21): Available V2G reserve

re,tV2G   is the available V2G reserve after the calculation, by Eq (21) of the capacity that can be comitted only if the EV keeps enough charging level to secure the schedule trip distances and the corresponded energy consumption. It is the maximum power V2G that can be comitted by the EV.

re,tV2G≤minPedisch,max,  Ee,tEV- Ee,tres +*nedischΔt *αe,n,t*1 (me=2) (21)

Literature basis: (Kempton & Tomic, 2005);  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026).

## Eq. (22): Activated V2G energy

To separate the paid availability from actual battery dispatch, the activated V2G energy is given by Eq (22):

qe,n,tV2G, actIV= αe,tV2G Δt (22),

with 0≤αe,tV2G≤re,tV2G

αe,tV2Gis the activated discharge power; re,tV2G is the maximum committed reserve, so activation cannot exceed it.

Literature basis: (Kempton & Tomic, 2005);  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026).

## Eq. (23): No simultaneous charging and discharging

Pᶜʰe,t≤zᶜʰe,t*Pᶜʰ,ᵐᵃˣe (a)

Pᵈⁱˢe,t≤zᵈⁱˢe,t*Pᵈⁱˢ,ᵐᵃˣe (b)

zᶜʰe,t+zᵈⁱˢe,t≤1 (c)

zᶜʰe,t , zᵈⁱˢe,t ∈{0,1 } (d)

Where zᶜʰe,t is the Binary variable indicating whether EV is in charging mode during period t and zᵈⁱˢe,t is the Binary variable indicating whether EV is in discharging mode during period t.

These 2 binary variables define the operation status of the EV battery.

| zᶜʰe,t | zᵈⁱˢe,t | Operating mode | Permissible? |
| --- | --- | --- | --- |
| 0 | 0 | Idle | Yes |
| 1 | 0 | Charging | Yes |
| 0 | 1 | Discharging | Yes |
| 1 | 1 | Simultaneous charging and discharging | NO |

Binary charging and discharging indicators are introduced to enforce mutually exclusive EV battery operating states. The corresponding power constraints limit charging and discharging to their respective rated capacities, while the mutual exclusivity condition prevents simultaneous charging and discharging within the same time interval. This formulation ensures physically consistent battery operation and supports the optimization of EV flexibility in local P2P electricity markets and V2G services.

Literature basis:  (Parisio, Rikos, & Glielmo, 2014).

## Eq. (24): Multi-choice mode restrictions

me=0⇒qe,n,tP2P,buy=qe,n,tP2P,sell=re,tV2G=0me=1⇒qe,n,tP2P,sell=re,tV2G=0me=2⇒qe,n,tP2P,buy,qe,n,tP2P,sell,re,tV2G may be positive if feasible

The multi-choice mode restrictions, mapping the EV’s discrete strategic choice into operational constraints are given by Eq (24):

m=0, no participation of EV in local market P2P. If charging is necessary for EV, then it can be done through the gridm=1, smart charging through local P2P purchase available but not the selling to the P2P market or V2G participationm=2, smart charging, local P2P buying or selling and V2G participation

Literature basis:  (Mailath, 2019);  (Vohra, 2011).

# Nodal balance, P2P reservation values, and bargaining

## Eq. (25): Net nodal exchange

In case of the net nodal exchange, the net electricity position of each node is calculated, before any external balancing. Positive means net supply and negative means net demand.

Xn,t= Gn,t+ Pn,tB,disch+Pn,tEV,disch -Ln,t- Pn,tB,ch-Pn,tEV,ch Eq (25)

Term interpretation: Xn,t is the node’s net supply: PV generation and battery/EV discharge are positive; building load and battery/EV charging are negative.

Literature basis:  (Long, Wu, Zhou, & Jenkins, 2017);  (Parisio, Rikos, & Glielmo, 2014).

## Eq. (26)  |  M-EQ26: Community energy balance

In case of Community energy balance, an optimizer equality enforces the hourly conservation of energy across local trades and the external grid.  The energy that is injected from the grid or through p2p net, needs to meet (cover) the net demand of all actors.

nPn,tgrid+- Pn,tgrid- + bqb,tp2p, buy- sqs,tp2p, sell= n(Ln,t+Pn,tB,ch+ Pn,tEV,ch- Gn,t- Pn,tB,disch- Pn,tEV,disch ) Eq (26).

In the left part of the equation:

Pn,tgrid+ , injection from the grid

Pn,tgrid-, export to the grid (extracted from the community)

qb,tp2p, buy , energy that the buyer buys from the p2p network

qs,tp2p, sell , energy that the sellers sells to the p2p network

In the right part of the equation net demand is represented: building (prosumers) load, battery charging, minus PV generation and discharge available energy.

Attention has to be kept in the unit conversion. P is power in kW and q are energy quantities in kWh, therefore all terms need to be transformed in the same unit system, actually P have to be multiplied by the hourly time window Δt.

Literature basis:  (Long, Wu, Zhou, & Jenkins, 2017);  (Parisio, Rikos, & Glielmo, 2014).

## Eq. (27): Prosumer reservation price

The prosumer reservation price defines the prosumer's minimum acceptable P2P price (as a seller):

pn,tseller,min=max⁡{λtsell ,  CnPV+ Cn,tB,deg } (27)

Where λtsell , the revenue from the selling of prosumer’s energy to the grid (the alternative option of the prosumer) and CnPV+ Cn,tB,deg ,  the operational costs derived from the operation of PV and degradation of prosumer’s battery (residential or commercial building).

Literature basis:  (Paudel, Chaudhari, Long, & Gooi, 2019);  (Vohra, 2011).

## Eq. (28): EV buyer reservation price

pᵇ,ᵐᵃˣe,t=min{λᵇᵘʸt,WTPe,tEV}

Purpose: Caps the P2P price at the cheaper of grid charging and the EV owner's willingness to pay.

Term interpretation: pᵇ,ᵐᵃˣe,t is the EV buyer’s maximum acceptable price; ᵇᵘʸtis the grid-buy alternative; WTPe,tEV is the EV owner’s willingness to pay.

Literature basis:  (Mailath, 2019);  (Vohra, 2011).

## Eq. (29): Bilateral Nash-bargaining P2P price

ptp2p=pptseller, min+βtptbuyer, max-ptseller, min,0≤βt≤1

Purpose: Splits the feasible price interval according to bargaining power.

Term interpretation: ptp2p is the negotiated P2P price; ptseller, min and ptbuyer, max are the seller’s minimum and buyer’s maximum prices; β_t∈[0,1] represents the bargaining weight assigned above the seller’s reservation price.

Literature basis:  (Nash, J. F. , 1950);  (Paudel, Chaudhari, Long, & Gooi, 2019).

## Eq. (30): Price-feasibility and bargaining sensitivity

qtp2p>0 only if ptbyuer,max≥ptseller, min  (a), trading feasibility condition

∂ptp2p∂βτ= ptbyuer,max- ptseller, min (b), P2P price sensitivity to the bargaining parameter

P2P trading is economically feasible only when the buyer's maximum willingness to pay is at least equal to the seller's minimum acceptable price. Within this feasible price interval, the bargaining parameter determines the allocation of the trading surplus between the two parties. The sensitivity of the negotiated P2P price to the bargaining parameter equals the difference between the buyer's and seller's reservation prices, assuming these prices remain constant.

Literature basis:  (Mailath, 2019);  (Nash, J. F. , 1950);  (Vohra, 2011).

## Eq. (31a) / Eq. (31b) / Eq. (31c): Hourly gains from P2P trade

Regarding the bilateral trading surplus, Eq (31) represents the hourly gains from P2P trade, separating total trade surplus into buyer and seller gains.

GStotal= GStbuyer+ GStseller=[ ptbuyer, max- ptseller, min ]*qtp2p (31a)

GStbuyer=ptbuyer, max- ptp2p *qtp2p  (31b)

GStseller=ptp2p- ptseller,min *qtp2p (31c)

The mutual beneficial transaction should be encouraged, therefore: ptbuyer, max≥ ptseller, min

Literature basis:  (Nash, J. F. , 1950);  (Paudel, Chaudhari, Long, & Gooi, 2019).

# Stakeholder payoffs, V2G compensation, and welfare

## Eq. (32): V2G availability payment

V2G availability payment defines payments for reserved bidirectional capacity even when no discharge occurs:

Re,tavail= Πtavail*re,tV2G*Δt (32)

Where Re,tavail, the revenue for the availability of EV to commit V2G service

Πtavail, the price for this availability

re,tV2G, the power for the EV commitment for V2G availability

Δt time duration (hours)

Literature basis: (Kempton & Tomic, 2005);  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026).

## Eq. (33): V2G activation payment

The payment compensates the commitment of capacity as available, even if discharge is not ultimately requested. Payment for energy that is actually activated is calculated separately as activation revenue (Eq 33).

Re,tactiv= Πtactiv*qe,tV2G, activ (33)

Literature basis: (Kempton & Tomic, 2005);  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026).

## Eq. (34): Minimum acceptable V2G remuneration

The EV owner’s private payoff accounts for mobility utility, charging expenditure, P2P and V2G revenues, battery degradation, inconvenience, and participation costs.

Minimum acceptable V2G remuneration, ensuring that V2G compensation covers private costs plus the required participation premium, is calculated by the Eq (34):

Re,tavail+ Re,tactual ≥ Ce,tdeg+ Ce,tinconv+Ce,tpartic+ ρe (34)

Where Ce,tdeg the degradation cost of EV battery

Ce,tinconv Inconvenience cost

Ce,tpartic the participation cost

ρe , the required participation premium (assumed 0,10 € per participation)

Literature basis:  (Mailath, 2019);  (Vohra, 2011).

## Eq. (35): EV battery-degradation cost

Ce,tdeg= cedeg*Pe,tch+ Pe,tdisch *Δt (a), in €

cedeg=Cereplac2*BeEV*Necyc (b)

Where cedeg is the unit cost of degradation in €/Kwh, Cereplac is the replacement cost of EV battery, BeEV is the energy corresponding to the nominal capacity and Necyc is the EV battery lifetime in equivalent cycles.

EV battery degradation costs are approximated using a linear energy-throughput model, in which the total charging and discharging energy is multiplied by a unit degradation cost. The latter is estimated by allocating the battery replacement cost over its expected lifetime energy throughput, expressed in equivalent full charge–discharge cycles. This formulation internalizes battery wear into the economic optimization of EV charging, discharging, and P2P energy trading decisions.

NTB=TR-EPC-BDC (c), where

NTB : Net Trading Benefit, TR = Trading Revenue, EPC= Energy Procurement Cost, BDC=Battery Degradation Cost

Purpose: Values cycling using energy throughput and replacement cost.

Literature basis:  (Bishop, J. D. K., et al., 2013).

## Eq. (36): EV inconvenience cost

Ce,tinc= ζe2* [Ee,tpref-Ee,tEV ]+2, if Ee,tEV<Ee,tpref0, if Ee,tEV≥Ee,tpref

This equation describes the inconvenience cost (or utility loss), which the EV owner face, when the restored energy of the EV battery is lower than the desirable level. The basic idea is that the EV owner might want to maintain the charge state in a specific level, even this level is not absolutely necessary for the EV’s mobility.

Ce,tinc is EV owner's inconvenience cost associated with insufficient stored battery energy; ζe is the inconvenience cost coefficient reflecting the EV owner's sensitivity to energy shortfalls; Ee,tpref preferred battery energy level of EV during period t; Ee,tEV is the actual stored energy in the EV battery during period t.

To explain the quadratic term, the more the EV battery energy is reduce, the greatest the EV owner’s inconvenience becomes. The marginal increase of inconvenient cost with regards to the energy deficit De,t=  [Ee,tpref-Ee,tEV ]+  becomes:  ∂Ce,tinc∂De,t= ζe*De,t (a).

Eq (36) introduces the behavioral dimension of EV owner into the economic model. The Departure energy adequacy constraint is a mandatory technical constrain, ensuring the sufficient energy for scheduled mobility needs, while the EV inconvenience cost is the economic evaluation of EV owner’s preference. In other words, the EV may be technically capable to sell the energy, but this transaction might not make economic sense, when inconvenience cost is integrated.

Literature basis:  (Dimonti, N. & Catchpole, K., 2025);  (Samadi, P., Mohsenian-Rad, A., Schober, R., Wong, V. W. S., & Jatskevich, J., 2010);  (Small & Verhoef, 2007).

## Eq. (37: Fixed and perceived participation cost

Cᵖᵃʳᵗe,s=Fᶜᵒⁿᵗe+Cᵈᵃᵗᵃe+Cᵗʳᵃⁿˢe+Cʳⁱˢᵏe

Purpose: Represents contracting, privacy, transaction and perceived range-risk costs per participation session.

Term interpretation: Cᵖᵃʳᵗe,s is participation cost for session s; Fᶜᵒⁿᵗ is contracting cost; Cᵈᵃᵗᵃ is data/privacy cost; Cᵗʳᵃⁿˢ is transaction cost; Cʳⁱˢᵏe is perceived range-risk cost.

Literature basis:  (Dimonti, N. & Catchpole, K., 2025);  (Mailath, 2019).

## Eq. (38): EV-owner net benefit

The resulting EV-owner payoff, measuring the EV owner's mobility-aware private economic outcome relative to the chosen baseline, is calculated in Eq (38):

ΠeEV= t[ Ue,tmob+ Re,tp2p+ Re,tV2G- Ce,tcharge- Ce,tdeg- Ce,tinc]- sCe,tpartic (38)

Where Ue,tmob is the Mobility-service utility, meaning the value to EV owner e of maintaining sufficient vehicle availability and battery energy to meet scheduled travel requirements during period t.

Re,tp2p , the revenues from selling energy through p2p market.

Re,tV2G, the revenues from selling energy through V2G option

Ce,tcharge, is the cost of charging through grid or p2p market

The first sum term includes all the hourly profits and costs, while the second sum term removes the cost per participation.

Participation is voluntary. In our model S is the alternative smart charging option, by purchasing with the grid, without any participation in local P2P market or V2G option. The EV owner’s payoff under a participating mode is checked against the strongest feasible outside option, including the specified grid-only or smart grid-only baseline.

Literature basis:  (Dimonti, N. & Catchpole, K., 2025);  (Mailath, 2019);  (Small & Verhoef, 2007).

## Eq. (39): Prosumer net benefit

Prosumer’s net benefit is calculated in Eq (39);

Πnprosumer= t[ Rn,tp2p+ Rn,tgrid,export+ Cn,tavoided- Cn,tBESS,deg- Cn,toper- Cn,tgrid, import ] (39)

## Eq. (40): DSO/grid net benefit

Πᴰ=Rⁿᵉᵗ+Cᵃᵛᵒⁱᵈ,ᵖᵉᵃᵏ+Cᵃᵛᵒⁱᵈ,ᶜᵒⁿᵍ+Cᵃᵛᵒⁱᵈ,ˡᵒˢˢ-Cⱽ²ᴳ,ᵃᵛᵃⁱˡ-Cⱽ²ᴳ,ᵃᶜᵗ

The DSO/grid payoff is calculated in Eq (40).

ΠDSO= Rnet+ Cavoided, peak+ Cavoided, congesion+ Cavoided, loss- CV2G, avail- CV2G, activ (40)

The accounts retain revenues, expenditures, avoided costs, and operating costs as separate components.

Literature basis: (Kempton & Tomic, 2005);  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026).

## Eq. (41): Hourly grid-related emissions

CO2,t=EFt*n​Pn,tgrid+Δt

Purpose: Calculates emissions using the hourly marginal or average grid-emission factor selected for the study.

Term interpretation: CO2,tis grid-related emissions in hour t; EFt is the selected grid-emission factor; Pn,tgrid+ is grid-import power

Literature basis:  (Hawkes, A. D. , 2010);  (2006 IPCC guidelines for national greenhouse gas inventories: Volume 2, Energy, 2006).

## Eq. (42): Monetized carbon externality

DtCO2=(ptcarb*CO2,t)/1000

Purpose: Values tonnes of CO₂ at the selected carbon price.

Term interpretation: DtCO2 is the monetised carbon externality; ptcarb is the carbon price in (EUR/tCO₂).

Literature basis:  (Commission, n.d.);  (Fabra, N. & Reguant, M., 2014).

## Eq. (43): Real system cost

Csys=t​Ctgrid+Ctdeg+Ctinc+Ctnet+Ctop+DtCO2+Ctuns

Purpose: Aggregates resource costs and externalities while excluding internal P2P/V2G transfers.

Term interpretation: Cˢʸˢ is real system cost; its terms include grid procurement, degradation, inconvenience, network and operating costs, monetised carbon externality, and unserved-energy cost. Internal P2P/V2G transfers are excluded.

Literature basis:  (Hawkes, A. D. , 2010);  (Nash, J. F. , 1950);  (Samadi, P., Mohsenian-Rad, A., Schober, R., Wong, V. W. S., & Jatskevich, J., 2010).

## Eq. (44): Social welfare

SW=e​Uemob+n​Unenergy-Csys

Social welfare is defines as mobility and energy-service utility net of real system costs:

SW= eUemobil+ nUnenergy- Creal, cost system (44)

eUemobil , is the overall mobility utility for all the EV owners (in our case only 1 owner)

nUnenergy , the utility of the energy service by all participants

Creal, cost system including generation PV and operation, degradation for batteries costs, losses costs.

Literature basis:  (Samadi, P., Mohsenian-Rad, A., Schober, R., Wong, V. W. S., & Jatskevich, J., 2010);  (Small & Verhoef, 2007).

## Eq. (45): Cancellation of internal transfers

a​PaymentsaP2P=0, a​PaymentsaV2G=0

Purpose: Prevents double-counting monetary transfers as newly created social value.

Term interpretation: These payments recorded across stakeholders α. Within a closed stakeholder set, each internal payment has an equal payer-side and recipient-side entry and therefore sums to zero.

Literature basis:  (Nash, J. F. , 1950);  (Samadi, P., Mohsenian-Rad, A., Schober, R., Wong, V. W. S., & Jatskevich, J., 2010).

# Fuzzy multi-choice cooperative game and allocation

## Eq. (46): Coalition characteristic function

vS=CᴮᴸS-CᶜᵒᵒᵖS+RVS+CBS

Purpose: Measures the incremental value created by coalition S compared with its non-cooperative baseline.

Term interpretation: v(S) is the incremental value of coalition S; Cᴮᴸ(S) is its baseline cost; Cᶜᵒᵒᵖ(S) is its cooperative cost; RV(S) is resilience value; CB(S) is carbon benefit.

Literature basis:  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008);  (Shapley, L. S. , 1953).

## Eq. (47): Fuzzy multi-choice coalition value

vS,μ,m=μSvS,m, μS=A{μi:i∈S}, mi∈{0,1,2}

Purpose: Combines coalition membership certainty/willingness with each EV's discrete participation mode.

Term interpretation: vS,μ,m is fuzzy coalition value; μSvS,m is the coalition’s aggregated degree of participation/availability under operator A; vS,m is the coalition value for participation-mode profile m; each mi is a permitted discrete mode.

Literature basis:  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008).

## Eq. (48): Shapley value

Shapley value is given by Eq (48), calculating the share of overall co-operative value, thanks to participant i, based on its own contribution in all possible cooperative cases.

φi (U)=S⊆N\{i}S ! N- S-1!N !*[USU{i}-U(S)] (48)

Where S is a subset of N, U(S) , the value created by S subset, and the term

[USU{i}-U(S)] , meaning the added value when participant I is entering subset S;

S ! N- S-1! Counting the possible orderings, in which the rest of the participants can come before or after the participant i, in the order in which participants join a coalition.

Literature basis:  (Shapley, L. S. , 1953).

## Eq. (49a) / Eq. (49b): Utopia payoff and minimum right

The limits of distribution in the calculation of t-value, are building the upper and lower payoff bounds required for the τ-value, therefore:

The upper limit for the share of participant i, is given by:

Mi=UN-U(N\{i}) (49a)

Where U(N\{i} is the value of the coalition since participant i, is outside this coalition.

The low limit for the share of participant i, is given by:

mi= maxS∋i[U(S)- j∈S  \{i}Mj ] (49b)

j∈S  \{i}Mj , means that the share of the participant i is not included (only the shares of the rest of the participants).

Mi is the contribution of participant i, in the overall set of the great coalition N. It compares the value of all participants with the value of the alliance that excepts participant i.

mi is the minimum right of participant i. It examines every alliance that includes this participant, and then it calculates how much value is remained for this participant, after removing the maximum shares Mj of the rest of the participants. It keeps for him the highest value among the others.

Literature basis:  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008);  (Tijs, S. H, 1981).

## Eq. (50a) / Eq. (50b): τ-value

τi=mi+αMi-mi, α=vN-i​mii​Mi-i​mi

The calculation of τ-value selects a compromise allocation between minimum rights and utopia payoffs for quasi-balanced games.

τi=mi+a*(Mi- mi) (50a)

a= UN- i∈Nmii∈NMi- i∈Nmi (50b)

Where α coefficient defines how far from the low limit the share will move towards the upper limit. The allocation should be between the 2 limits, therefore: 0<α<1. This will become the allocation of the cooperative value and the U(N) is the added value of the alliance, in comparison to the no-cooperative scenario.

Literature basis:  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008);  (Tijs, S. H, 1981).

## Eq. (51): Core feasibility

i∈N​xi=vN, i∈S​xi≥vS ∀S⊂N

Purpose: Tests whether an allocation is efficient and coalition-stable.

Term interpretation: xi is the allocation to player i; vN is the grand-coalition worth; vS is the worth of coalition S; efficiency requires total allocation to equal vN, while coalition stability requires each coalition to receive at least its worth.

Literature basis:  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008); (Potters, J. A. M., Poos, R., Tijs, S. H., & Muto, S, 1989) ;  (Shapley, L. S. , 1971).

## Eq. (52)  |  M-EQ52: Average lexicographic (AL/Alexia) allocation

the AL allocation averages the lexicographic core allocations obtained across all player orderings, producing the AL/Alexia value:

XiALU= 1Ν! σ∈Π(Ν)Liσ(U) (52)

Where XiALU is the share of participant i.

N is the population of participants

Ν! is the number of possible orderings in which the participants can be placed

Π(Ν) is the set of these orderings, of possible relocations σ

Liσ(U) is the share of participant i, in the specific allocation that is derived for the specific ordering σ. Therefore, for every possible ordering, the first participant maximizes its own share, then the second in order participant etc. For each participant i, the average of its shares from all orderings is calculated. To be more specific, in our model, the cooperative value is allocated among EV owner, prosumer and DSO with 3! = 6 possible orderings.

The DSO-oriented clan formulation restricts the set of admissible coalitions according to the selected clan-game definition. General cooperative-game and clan-game results should be reported as distinct formulations, with the coalition restrictions and allocation feasibility checks stated for each.

Literature basis:  (Kongo, T., Funaki, Y., Brânzei, R., & Tijs, S, 2010);  (Tijs, S., Borm, P., Lohmann, E., & Quant, M., 2011).

# Resilience stress test

## Eq. (53): Expected energy not served

EENSs=n,t∈s​Lᵘⁿˢn,t*Δt

Purpose: Measures unserved electricity during each outage or congestion stress event.

Term interpretation: EENSs is expected energy not served during event s; n,t∈s​Lᵘⁿˢn,t is unserved load at node n and hour t.

Literature basis:  (Decision No 23/2020 on the methodology for calculating the value of lost load, the cost of new entry and the reliability standard, 2020).

## Eq. (54)  |  M-EQ54: Outage cost

Cᵒᵘᵗs=n,t∈s​VOLLn⋅Lᵘⁿˢn,tΔt

Purpose: Monetises interruption damage using stakeholder-specific Value of Lost Load.

Term interpretation: Cᵒᵘᵗ_s is outage cost for event s; VOLL_n is the value of lost load at node n; Lᵘⁿˢ is unserved load; Δt converts to unserved energy.

Literature basis:  (Ramanathan & Bharadwaj, 2025); (European Union Agency for the Cooperation of Energy Regulators, 2020)

## Eq. (55): Economic value of V2G resilience

The economic value of V2G resilience is calculated, with values avoided interruption cost net of V2G activation and degradation costs:

RVS= CSoutage, baseline- CSoutage, V2G- CSV2G,activ (55)

Where CSoutage, baseline is the cost of outage without V2G

CSoutage, V2Gis the cost of outage with V2G

CSV2G,activ is the cost of activation of the V2G energy.

Net profit is possible with RVS>0, while for RVS<0 the cost of energy activation exceeds the cost of the interruption that was avoided.

The stress-test results should identify the event definition, duration, affected demand, assumed VOLL, and whether EV energy is available at the relevant location and time. These assumptions should be reported alongside the results

Literature basis:  (Koubar, Fachrizal, Ericsson, Zabian, & Munkhammar, 2026);  (Ramanathan & Bharadwaj, 2025).

# KPIs, price transmission, and aggregation

## Eq. (56): Equivalent full cycles

EFCe=t​Pe,tch+Pe,tdisΔt2BeEV

Purpose: Reports battery use on a comparable cycle-equivalent basis.

Term interpretation: EFCe is EV equivalent full cycles; Pe,tch and Pe,tdisˢ are charging and discharging power; BeEV is battery capacity. The denominator counts a full cycle as charge-plus-discharge throughput equal to twice capacity.

Literature basis:  (Bishop, J. D. K., et al., 2013).

## Eq. (57): PV self-consumption ratio

SCRm=t∈m​Eᴾⱽ,ˢᵉˡᶠt/t∈m​GtΔt

Purpose: Measures the share of PV generation used locally in each month or year.

Term interpretation: SCRm is the PV self-consumption ratio for period m; Eᴾⱽ,ˢᵉˡᶠ is locally consumed PV energy; G_t is PV generation; the denominator is total PV energy in the period.

Implementation: Excel ratio of monthly sums; do not average hourly ratios.

## Eq. (58): Self-sufficiency ratio

SSRm=t∈m​Eˡᵒᶜᵃˡ,ˢᵘ*ᵖtt∈m ​L,tot,t *Δt

Purpose: Measures the share of demand supplied by local PV, BESS and EV discharge.

Term interpretation: SSRm is the self-sufficiency ratio; Eˡᵒᶜᵃˡ,ˢᵘᵖ_t is locally supplied energy; L,tot,t is total demand; the denominator is total demand in the period.

## Eq. (59)  : Volume-weighted P2P price

The model records hourly P2P prices and volumes. Monthly and annual weighted (based on the energy quantities purchased) transaction prices are calculated as volume-weighted prices, as an accounting convention, specified in Eq (59).

pmonthlyp2p= t∈m( ptp2p* qtp2p)t∈mqtp2p (59)

Literature basis:  (Long, Wu, Zhou, & Jenkins, 2017);  (Paudel, Chaudhari, Long, & Gooi, 2019).

## Eq. (60): Individual-rationality condition

Participation is voluntary. In our model S is the alternative smart charging option, by purchasing with the grid, without any participation in local P2P market or V2G option. The EV owner’s payoff under a participating mode is checked against the strongest feasible outside option, including the specified grid-only or smart grid-only baseline. Participation is treated as individually rational only when the participating payoff is at least as high as this outside-option payoff. This check is implemented using Eq (60):

ΠeEV m,  μ≥max{ ΠeEV m=0, ΠeEV (S)} (60)

Where ΠeEV m=0, the profit from no participation in p2p local markets, participating only in conventional (not smart) charging through the grid.

ΠeEV (S) , the profit from no participation in p2p local markets, participating only in smart (not conventional) charging through the grid. The max{ } option gives the best of the 2 alternative options.

Literature basis:  (Mailath, 2019);  (Vohra, 2011).

## Eq. (61): Wholesale-to-local price transmission

PT=CovptP2P,λtW/VarλtW; ptP2P=α0+α1λtW+α2ctnet+α3ptcarb+εt

Purpose: Quantifies how wholesale prices, network charges and carbon prices transmit into local P2P prices.

Term interpretation: PT is the wholesale-to-local price-transmission measure; Cov and Var are covariance and variance; ptP2P is local P2P price; λtW is wholesale price; ctnet is network charge; ptcarb is carbon-price signal; α₁ measures the wholesale-price coefficient; εt is the residual.

Literature basis:  (Fabra, N. & Reguant, M., 2014).

## Eq. (62): Temporal aggregation

Xm=t∈m​Xt;Xy=t=18760Xt

Purpose: Aggregates hourly energy, cost, revenue, welfare and emission results into monthly and annual outputs.

Term interpretation: Xm and Xy are the monthly and annual totals of an hourly variable Xt; m is the set of hours in the month; the annual sum covers all 8,760 hours.

# General theoretical references

The cooperative-allocation equations are interpreted within the transferable-utility cooperative-game tradition, including coalition values, allocation rules, core stability, and lexicographic-core selections. These references provide general theoretical foundations and do not imply additional properties beyond those explicitly tested in the model  (Brânzei, R., Dimitrov, D., & Tijs, S, 2008);  (Driessen, T. S. H, 1988);  (Moulin, H. , 1991).

# References

2006 IPCC guidelines for national greenhouse gas inventories: Volume 2, Energy. (2006).

Bishop, J. D. K., Axon, C. J., Bonilla, D., Tran, M., Banister, D., & McCulloch, M. D. (2013). Evaluating the impact of V2G services on the degradation of batteries in plug-in electric vehicles. Applied Energy, 111, 206–218.

Brânzei, R., Dimitrov, D., & Tijs, S. (2008). Models in cooperative game theory. Springer.

BS EN 15316-4-3:2017. (2026). https://assets.publishing.service.gov.uk/media/69a6cc7d2e1f4fbda42521fa/hem-tp-18-pv-generation-and-self-consumption.pdf. Modelling PV electricity generation and calculating self-consumption within the Home Energy . Department for Energy Security & Net Zero.

Commission, E. (n.d.). EU Emissions Trading System (EU ETS): Market and policy information.

Decision No 23/2020 on the methodology for calculating the value of lost load, the cost of new entry and the reliability standard. (2020).

Dimonti, N., & Catchpole, K. (2025). Evaluation of techno-economic and social aspects of centralised electric vehicle charging coordination strategies in residential settings. EEE International Conference on Energy Technologies for Future Grids. IEEE.

Driessen, T. S. H. (1988). Cooperative games, solutions and applications. Kluwer Academic Publishers.

ENTSO-E Transparency Platform [Data set]. (2025). Retrieved from https://transparency.entsoe.eu

European Union Agency for the Cooperation of Energy Regulators. (2020). Decision No 23/2020 on the methodology for calculating the value of lost load, the cost of new entry and the reliability standard,. EU, EU.

Fabra, N., & Reguant, M. (2014). Pass-through of emissions costs in electricity markets. American Economic Review, 104(9), 2872–2899.

Greek Ministry of Energy. (2024). National Energy and Climate Plan (NECP). National Energy and Climate Plan (NECP). Athens: Greek Ministry of Energy.

Hawkes, A. D. . (2010). Estimating marginal CO₂ emissions rates for national electricity systems. Energy Policy, 38(10), 5977–5987.

Karamanos, Dimara, A., Arvanitis, K., Timpalexis, C., Krinidis, S., & Tzovaras, D. (2019). Energy Consumption Patterns of Residential Users: A Study in Greece. Springer International Conference on Computer Vision Systems (pp. 639-650). Springer Nature Link Availabe: https://link.springer.com/chapter/10.1007/978-3-030-34995-0_58.

Kempton, W., & Tomic, J. (2005). Vehicle-to-grid power fundamentals: Calculating capacity and net revenue. Journal of Power Sources, 144(1), 268-279.

Kongo, T., Funaki, Y., Brânzei, R., & Tijs, S. (2010). Non-cooperative and axiomatic characterizations of the average lexicographic value. International Game Theory Review, 12(4), 417–435.

Koubar, M., Fachrizal, R., Ericsson, M., Zabian, O., & Munkhammar, J. (2026). Economic estimation of vehicle-to-grid potential for a solar prosumer office building: Optimization on day-ahead and manual restoration reserve markets. 22nd International Conference on the European Energy Market. IEEEXplore.

Koundouri, Alamanos, A., Arampatzidis, I., Dellis, K., Deranian, C., & Devves, S. (2025). Assessing national climate-neutrality plans through a modelling nexus lens: the case of Greece. npj Climate Action, 4.

Koundouris, Iliopoulou, Dimitriadis, & Mammasis. (2017). Investigation on the stochastic nature of the solar radation process. Energy Procedia, 125, 398-404.

Long, C., Wu, J., Zhou, Y., & Jenkins, N. (2017). Peer-to-peer energy trading in a community microgrid. IEEE Power & Energy Society. IEEE.

Mailath, G. J. (2019). Modeling strategic behavior: A graduate introduction to game theory and mechanism design. (L. N. Policy, Ed.) World Scientific.

Moulin, H. . (1991). Axioms of cooperative decision making. Cambridge University Press.

Nash, J. F. . (1950). The bargaining problem (Vol. 18(2)). Econometrica.

Palma-Behnke, R., Benavides, C., Lanas, F., Severino, B., Reyes, L., Llanos, J., & Sáez, D. (2013). A microgrid energy management system based on the rolling horizon strategy. IEEE Transactions on Smart Grid, 4(2), 996-1006.

Parisio, A., Rikos, E., & Glielmo, L. (2014). A model predictive control approach to microgrid operation optimization. IEEE Transactions on Control Systems Technology, 22(5), 1813–1827.

Paudel, A., Chaudhari, K., Long, C., & Gooi, H. B. (2019). Peer-to-peer energy trading in a prosumer-based community microgrid: A game-theoretic model. IEEE Transactions on Industrial Electronics, 66(8), 6087–6097.

Potters, J. A. M., Poos, R., Tijs, S. H., & Muto, S. (1989). Clan games. Games and Economic Behavior, 1(3), 275–293.

Ramanathan, N. S., & Bharadwaj, P. (2025). Value of lost load computation framework for vehicle-to-load applications considering life cycle techno-economic analysis. EEE 4th International Conference on Smart Technologies for Power, Energy and Control. IEEE.

Samadi, P., Mohsenian-Rad, A., Schober, R., Wong, V. W. S., & Jatskevich, J. (2010). Optimal real-time pricing algorithm based on utility maximization for smart grid. First IEEE International Conference on Smart Grid Communications, (pp. 415–420).

Shapley, L. S. . (1953). A value for n-person games. In A value for n-person games (pp. 307–317). Princeton University Press.

Shapley, L. S. . (1971). Cores of convex games. RAND Paper P-4620.

Small, K. A., & Verhoef, E. T. (2007). The economics of urban transportation. (K. A. Small, Ed.) Routledge. Taylor & Francis Group.

Sortomme, E., & El-Sharkawi, M. A. (2011). Optimal charging strategies for unidirectional vehicle-to-grid. IEEE Transactions on Smart Grid, 2(1), 131–138.

Tijs, S. H. (1981). Bounds for the core of a game and the τ-value. In S. H. Tijs, Bounds for the core of a game and the τ-value. Tilburg University.

Tijs, S., Borm, P., Lohmann, E., & Quant, M. (2011). An average lexicographic value for cooperative games. European Journal of Operational Research, 213(1), 210–220.

Vehicle-to-grid power fundamentals: Calculating capacity and net revenue. (2005).

Vohra, R. V. (2011). Mechanism design: A linear programming approach. Cambridge : Cambridge University Press.